Networking Fundamentals

Personal Study Notes

Based on David Bombal's “Complete Networking Fundamentals Course — Your CCNA Start”

Covering Lectures 1–37

How to read this document

Each topic follows the same format — a short Definition, a worked Example, and a Note where something is easy to mix up, with tables used for anything comparative.

Headings tagged NEW are extra foundational topics added to fill gaps — they were not in the original notes. Everything else is my own notes from the course, reformatted for consistency.

# Contents

Part 1 — Networking Basics  (NEW)

1.1  What Is a Network?

1.2  Network Topologies

1.3  Common Networking Devices

1.4  Duplex: Half vs Full

1.5  Collision Domains vs Broadcast Domains

1.6  Cabling Basics

Part 2 — Reference Models

2.1  OSI Model (7 Layers)

2.2  TCP/IP Model (5 Layers)

2.3  Encapsulation & Decapsulation  (NEW)

Part 3 — MAC Addressing & Address Resolution

3.1  How Switches Learn MAC Addresses

3.2  How Hosts Learn MAC Addresses (ARP)

Part 4 — IP Addressing

4.1  What Is an IP Address?

4.2  Network Portion vs Host Portion  (NEW)

4.3  IPv4 Address Format

4.4  Private vs Public IP Addresses

4.5  Classful IP Addressing

4.6  Broadcast Addresses

4.7  Local Loopback Address

4.8  IPv4 Link-Local Addresses

4.9 DHCP: Automatic IP Assignment  (partly NEW)

4.10 DNS: Translating Names to IP Addresses  (NEW)

4.11  Subnet Masks & CIDR Notation  (NEW)

# Part 1 — Networking Basics

## 1.1  What Is a Network?  NEW

Definition: A network is two or more devices connected together so they can share resources (files, printers, internet access) and communicate.

| Type | Full Name | Typical Range | Example |
| --- | --- | --- | --- |
| PAN | Personal Area Network | A few metres | Bluetooth between a phone and earbuds |
| LAN | Local Area Network | One building / site | Home or office network |
| MAN | Metropolitan Area Network | A city | City-wide ISP network |
| WAN | Wide Area Network | Country / global | The Internet; links between branch offices |

## 1.2  Network Topologies  NEW

Definition: A topology is the way devices and connections are arranged — physically (actual cabling) or logically (how data actually flows).

| Topology | Description | Note |
| --- | --- | --- |
| Bus | All devices share a single cable | Legacy; one break can take down the whole segment |
| Star | All devices connect to one central device (a switch or hub) | The standard for modern LANs |
| Ring | Devices connected in a closed loop | Used by legacy Token Ring networks |
| Mesh | Every device connects to every other device | Very redundant, but expensive to cable |
| Hybrid | A mix of two or more topologies | Common in larger, real-world networks |

## 1.3  Common Networking Devices  NEW

Definition: The devices below are the building blocks of every network you'll see in this course — each one operates mainly at a particular OSI layer.

| Device | OSI Layer | What It Does | Typical Use |
| --- | --- | --- | --- |
| Hub | Physical (L1) | Repeats an incoming signal out of every other port; no MAC learning | Legacy LANs (rarely used today) |
| Switch | Data Link (L2) | Learns MAC addresses and forwards frames only to the correct port | Connecting PCs, printers, etc. within a LAN |
| Bridge | Data Link (L2) | Connects two LAN segments and filters traffic by MAC address | Splitting up a large collision domain |
| Router | Network (L3) | Connects different networks and routes packets using IP addresses | Connecting a LAN to the Internet |
| Access Point | Data Link (L2) | Gives wireless devices access to a wired LAN | Wi-Fi networks |
| Firewall | L3–L7 | Filters traffic in/out of a network based on security rules | Protecting a network's edge |
| Modem | Physical (L1) | Converts signals between your LAN and your ISP's line (cable/DSL/fibre) | Connecting a home network to the ISP |

Note: A hub is a “dumb” repeater — every bit that arrives on one port goes out every other port, so all hub ports share one collision domain. A switch behaves like a smart, multi-port bridge: it learns where each device is and sends frames only where they need to go (see Part 3).

## 1.4  Duplex: Half vs Full  NEW

| Mode | Meaning | Analogy |
| --- | --- | --- |
| Half Duplex | A device can send or receive at a given moment, but not both | A walkie-talkie — only one person talks at a time |
| Full Duplex | A device can send and receive at the same time | A phone call — both sides can talk at once |

Note: Modern switch ports run full duplex, which is a big part of why collisions are basically a non-issue on switched networks.

## 1.5  Collision Domains vs Broadcast Domains  NEW

Definition: A collision domain is a section of network where two devices transmitting at the same time can cause a collision (this matters on shared/half-duplex media).

Definition: A broadcast domain is a section of network where a broadcast frame sent by one device is seen by every other device in it.

| Device | Collision Domains | Broadcast Domains |
| --- | --- | --- |
| Hub | 1 (every port shares one) | 1 |
| Switch | 1 per port | 1 (unless split into VLANs) |
| Router | 1 per port | 1 per port |

Note: This is why routers are said to “break up” broadcast domains, while switches — by default — do not.

## 1.6  Cabling Basics  NEW

| Cable / Term | Description |
| --- | --- |
| UTP (Unshielded Twisted Pair) | The common copper cable for LANs, terminated with an RJ45 connector |
| Fibre optic | Carries data as light pulses; long distance, immune to electrical interference |
| Straight-through cable | Wired the same at both ends; connects unlike devices (e.g. PC to switch) |
| Crossover cable | Wires swapped at one end; connects like devices (e.g. switch to switch) |

Note: Most modern switch and NIC ports support Auto-MDIX, which automatically detects and adjusts for the cable type — so the straight-through vs crossover distinction matters less than it used to.

# Part 2 — Reference Models

## 2.1  OSI Model (7 Layers)

| No. | Layer | Protocols / Mediums | Devices / Examples | Description |
| --- | --- | --- | --- | --- |
| 1 | Physical | Ethernet / Wi-Fi | Hubs, repeaters | Transmits raw bitstreams as electrical, optical, or radio signals |
| 2 | Data Link | Ethernet II, PPP, HDLC | Switches, bridges | Manages MAC addressing, framing, and error detection |
| 3 | Network | IPv4 / IPv6 | Routers | Logical addressing; determines the best path through networks |
| 4 | Transport | TCP / UDP | — | Supports end-to-end communication between devices |
| 5 | Session | — | — | For simplicity, often grouped with the Application layer |
| 6 | Presentation | — | — | For simplicity, often grouped with the Application layer |
| 7 | Application | HTTP, HTTPS, FTP, TFTP, Telnet | — | Represents data to users; encodes and controls dialogue |

## 2.2  TCP/IP Model (5 Layers)

| No. | Layer | Protocols / Mediums | Devices / Examples | Description | PDU |
| --- | --- | --- | --- | --- | --- |
| 1 | Physical | Ethernet / Wi-Fi | Hubs, repeaters | Transmits raw bitstreams as electrical, optical, or radio signals | Bits |
| 2 | Data Link | Ethernet II, PPP, HDLC | Switches, bridges | Manages MAC addressing, framing, and error detection | Frames |
| 3 | Network | IPv4 / IPv6 | Routers | Logical addressing; determines the best path through networks | Packets |
| 4 | Transport | TCP / UDP | — | Supports end-to-end communication between devices | Segments |
| 5 | Application | HTTP, HTTPS, FTP, TFTP, Telnet | — | Represents data to users; encodes and controls dialogue | Data |

## 2.3  Encapsulation & Decapsulation  NEW

Definition: Encapsulation is the process of wrapping data in the appropriate header (and sometimes trailer) as it travels down the layers on the sending device. Decapsulation is the reverse — stripping those headers off as the data travels up the layers on the receiving device.

Example: An HTTP request becomes a Segment (Transport header added), then a Packet (IP header added), then a Frame (Ethernet header added) before it becomes Bits on the wire. The receiving device reverses each step.

Note: The PDU (Protocol Data Unit) column in the TCP/IP table above is just the name for “data at this layer” — Bits, Frames, Packets, Segments, Data — from the bottom up.

# Part 3 — MAC Addressing & Address Resolution

## 3.1  How Switches Learn MAC Addresses (MAC Address Table)

Definition: A switch builds and maintains a MAC address table. It learns addresses by inspecting the source MAC address of incoming traffic and recording it against the port it arrived on (if it isn't already recorded).

- If the destination MAC address is known, the switch forwards the frame only to that specific port.

- If the destination MAC address is unknown, the switch floods the frame out of every connected port except the one it arrived on. The intended device replies, while every other device ignores the frame; the switch then learns the destination's MAC address from that reply, using the same process.

## 3.2  How Hosts Learn MAC Addresses (ARP)

Definition: A host learns other devices' MAC addresses using ARP (Address Resolution Protocol):

- The host sends an ARP broadcast containing the destination's IP address onto the local network.

- If the network is connected via a switch, the switch forwards this broadcast to every other device.

- Every device except the intended target drops the packet; the target replies directly to the sender.

- On receiving the reply, the host learns the destination's MAC address.

# Part 4 — IP Addressing

## 4.1  What Is an IP Address?

Definition: An Internet Protocol (IP) address is used to route traffic across the internet. Every internet-connected device needs one, but because IPv4 addresses have run out, most home and office devices use a private IP address rather than a public one.

## 4.2  Network Portion vs Host Portion  NEW

Definition: Every IP address is split into a network portion (which network the device is on) and a host portion (which specific device on that network) — much like a street address.

Example: Think of “10 Downing Street”: “Downing Street” is the network (the street everyone on it shares), and “10” is the host (the specific house). For 192.168.1.10 /24, the network portion is 192.168.1 and the host portion is .10.

Note: How many bits belong to each portion is exactly what the subnet mask / CIDR notation defines — see section 4.6.

## 4.3  IPv4 Address Format

Definition: IP has two versions: IPv4 and IPv6.

- A 32-bit address, divided into four 8-bit octets.

- Example: 1.1.1.1

- Each octet ranges from 0–255.

## 4.4  Private vs Public IP Addresses

Definition: A private IP address is a non-routable identifier assigned to devices within a local network (e.g. a home or office LAN). It enables internal communication while staying hidden from the public internet. These ranges are defined by RFC 1918 (IPv4) and RFC 4193 (IPv6) to prevent conflicts with public internet addresses.

| Class | Range | Address Count |
| --- | --- | --- |
| A | 10.0.0.0 – 10.255.255.255 | ~16.7 million |
| B | 172.16.0.0 – 172.31.255.255 | ~1 million |
| C | 192.168.0.0 – 192.168.255.255 | 65,536 |

Any address outside these ranges is considered a public IP address.

Definition: A public IP address is a unique, globally routable identifier assigned by an Internet Service Provider (ISP), allowing a device or network to communicate directly over the internet. Unlike private IPs, public IPs are visible to the entire internet and are essential for accessing websites, hosting servers, and enabling remote connectivity.

## 4.5  Classful IP Addressing

Definition: Classful addressing splits the whole IPv4 range into classes, each with a fixed, default split between network bits and host bits.

| Class | Address Range | First Octet | Default Mask | CIDR | Hosts / Network | Purpose |
| --- | --- | --- | --- | --- | --- | --- |
| A | 0.0.0.0 – 127.255.255.255 | 0–127 | 255.0.0.0 | /8 | 16,777,214 | Large networks (govt, ISPs, big corporations) |
| B | 128.0.0.0 – 191.255.255.255 | 128–191 | 255.255.0.0 | /16 | 65,534 | Medium–large networks (universities, companies) |
| C | 192.0.0.0 – 223.255.255.255 | 192–223 | 255.255.255.0 | /24 | 254 | Small networks (offices, homes, small business) |
| D | 224.0.0.0 – 239.255.255.255 | 224–239 | N/A | N/A | N/A | Multicast groups (streaming, routing protocols) |
| E | 240.0.0.0 – 255.255.255.255 | 240–255 | N/A | N/A | N/A | Experimental / reserved for research |

Note: Classes A, B and C are unicast addresses; Class D is multicast; Class E is reserved for research.

## 4.6  Broadcast Addresses

Directed Broadcast Address

Used to send traffic to every device within a specific local network/subnet. All bits in the host portion of the address are set to binary 1s.

Example: Network ID 192.168.1.0 → Directed Broadcast Address 192.168.1.255 (host bits set to 1).

Local Broadcast Address

Used to communicate with every device on the local network/subnet. The entire address is binary 1s: 255.255.255.255.

Example: When a new device joins a network, it sends a local broadcast to request an IP address from a DHCP server (see 4.10).

Note: Layer 3 devices (e.g. routers) always drop this address — it never crosses a subnet boundary. If the DHCP server sits on a different network/VLAN, the device can't reach it directly, so a router forwards the request as a unicast DHCP request using DHCP Relay (a.k.a. DHCP Forwarding).

## 4.7  Local Loopback Address

Definition: Used by a device to send traffic to itself, primarily for testing and diagnostics (e.g. verifying the local TCP/IP stack is working correctly). Traffic sent to a loopback address never leaves the device — it is not transmitted onto the physical network.

|  | Value |
| --- | --- |
| IPv4 range | 127.0.0.0 – 127.255.255.255 |
| IPv6 address | ::1 |
| Most common address | 127.0.0.1 |

Note: Routers have their own “loopback interface”, which is a different concept from this local loopback address.

## 4.8  IPv4 Link-Local Addresses

Definition: Used when a PC is configured for DHCP but no DHCP server is available.

|  | Value |
| --- | --- |
| Range | 169.254.0.0/16 (mask 255.255.0.0) |

- Historically, it let two computers communicate with each other even with no DHCP server available.

- Devices can start using it immediately, with no configuration needed.

- Hosts randomly generate the host portion of the address.

Note: A link-local address is not routable on the internet or across different networks/subnets.

## 4.9  DHCP: Automatic IP Assignment  NEW

Definition: DHCP (Dynamic Host Configuration Protocol) automatically hands a host its IP configuration (address, subnet mask, default gateway, DNS servers) so nobody has to set it manually. The four-step exchange is often remembered as DORA:

- Discover — the client sends a local broadcast looking for a DHCP server.

- Offer — a DHCP server replies, offering an available IP address.

- Request — the client requests that specific offered address.

- Acknowledge — the server confirms the assignment (ACK).

Note: This is the same local broadcast referenced in section 4.7 — and DHCP Relay is what lets that broadcast reach a DHCP server on a different subnet.

## 4.10  DNS: Translating Names to IP Addresses  NEW

Definition: DNS (Domain Name System) resolves human-friendly domain names into the IP addresses computers actually use to communicate.

Example: Typing www.example.com into a browser causes a DNS lookup that returns something like 93.184.216.34, which is the address your device actually connects to.

Note: Without DNS, you would need to remember the numeric IP address of every website you wanted to visit.

## 4.11  Subnet Masks & CIDR Notation  NEW

Definition: A subnet mask marks which bits of an IP address are the network portion (1s) and which are the host portion (0s). CIDR notation (/n) is shorthand for the same thing — it just states how many bits, from the left, belong to the network.

Example: 255.255.255.0 and /24 mean exactly the same thing: the first 24 bits (3 octets) are the network portion, and the remaining 8 bits are the host portion.

Note: This is the same CIDR column already shown in the classful table above (/8, /16, /24) — subnetting (splitting these further) is typically covered in more depth later in the course.